Metrix OX8022 & OX8032

________________________________________________________________________________

Introduction:

This instrument driver provides programming support for Metrix OX8022 & OX8032.

It contains functions for opening, configuring, taking measurements from, and

closing the instrument.

________________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

For GPIB instrument drivers:

-  the instrument is connected to the GPIB.

-  the GPIB address supplied to the initialize function must match the

GPIB address of the instrument.

For VXI instrument drivers:

-  the instrument is installed in the VXI mainframe and you are

using one of the following controller options:

Embedded controller

AT-MXI

MC-MXI

GPIB-VXI

-  the logical address supplied to the initialize function must

match the logical address of the instrument.

-  if you are using GPIB-VXI, there must be at least one unused

device name in the handler.

For RS-232 instrument drivers:

-  the instrument is connected to the RS-232 interface.

-  the COM port, baud rate, parity, and timeout supplied to the

initialize function must match the settings of the instrument.

________________________________________________________________________________

Error Codes:

Error codes are returned as the return value of each instrument driver

function. A program should examine this value after each call to an instrument

driver function to determine if any error occurred. Possible error codes and

their meanings are listed with the corresponding instrument driver function.

________________________________________________________________________________

How To Use This Document:

This document is intended to be used as a programming reference manual.

It describes each function in the

Metrix OX8022 & OX8032

instrument. The functions are listed in alphabetical order with a

description of the function, C syntax of the function, a description of each

parameter, and possible error codes.

________________________________________________________________________________

Function Tree Layout:

Metrix OX8022 & OX8032                 Function Name:

Initialize                                  ox8022_init

Configure

Save/Recall Setup                        ox8022_setup

MODE & AUTOSET                           ox8022_cont_inst

Inputs                                   ox8022_channel

Channel Save                             ox8022_channel_save

Acquisition                              ox8022_acquire

Display                                  ox8022_display

Time Base                                ox8022_timebase

Trigger                                  ox8022_trigger

Analyse

Read Waveform                            ox8022_wave_read

Automatic Measurements                   ox8022_measure

IEEE 488.2 Comm/Queries                  ox8022_status

Instrument Queries                       ox8022_instrument_query

Close                                       ox8022_close

________________________________________________________________________________

Metrix OX8022 & OX8032

Instrument name: Metrix OX8022

Description: This instrument module provides

GPIB and RS232 support for OX8022.

The module contains the followings

functions:

(1) Initialize

Initialize the instrument module

and the communication.

(2) Configuration

Class of functions for configuring

the following parameters:

* Save/Recall Setup

* Mode && Autoset

* Channel Settings

* Channel Save

* Acquisition settings

* Display settings

* Timebase settings

* Trigger

(3) Analyse

Class of functions for configuring

the following parameters:

* Waveform Read

* Automatic Measurements

* IEEE 488.2 queries

* Instrument queries

(4) Close

Close the instrument module and

unlock the front panel .

________________________________________________________________________________

The following functions are in alphabetical order.

________________________________________________________________________________

ox8022_acquire

int ox8022_acquire (int MODE, int REFerence, int RUNSTOP, int PEAK, int WAI);

Purpose

Panel name  :   Acquisition settings

Description :   This panel is usefull to

configure and control the

acquisition process.

The following parameters

can be set:

* Acquisition MODE:SINGLE,REFRESH,ROLL

* Envelop mode (OFF or ON)

* Pre-trigger depth :  0..4ko

Parameter List

MODE

Variable Type       int

Control label  : Acquisition Mode

Description    : This control sets the

acquisition mode in single,

refresh or roll mode.

Variable type  : integer

Variable range : 0 : SINGLE

1 : REFRESH  (default value)

2 : ROLL

REFerence

Variable Type       int

Control label  : PRETRIG

Description    : this control sets the pretrig

value.

Variable type  : integer

Variable range :    -1      : Continuous ROLL

0 to 4 : 0k to 4ko

Default value  : 0k

RUNSTOP

Variable Type       int

Control label  : control

Description    : this parameter runs or

stops the acquisition process

Variable type  : integer

Variable range : 0 : Stop  (default value)

1 : Run

PEAK

Variable Type       int

Control label  : PEAK

Description    : This control valids or not

the envelop mode .

Variable type  : integer

Variable range : 0 : OFF   (default value)

1 : ON

WAI

Variable Type       int

Control label  : *WAI

Description    : When ON , the oscilloscope

waits for the end of acquisi-

tion before executing next

orders stored in its input

buffer . This function is

usefull when you need the

result of the acquisition

(for example : reading wave-

form or automatic measure-

ments) . Available with

Single only .

Variable type  : integer

Variable range : 0 : OFF   (default value)

1 : ON

Return Value

Control label  : Error

Description    :  Displays the result of the

function call.

Variable type  : integer

Variable range :

0  :   Success

-1  :   MODE out of range

-2  :   REFerence out of range

-3  :   PEAK out of range

-4  :   RUN/STOP out of range

-5  :   *WAI out of range

-6  :   ROLL not allowed with timebase

faster than 500mS/Div.

-7  :   REFerence -1 allowed with ROLL

-8  :   PEAK allowed with REFRESH only

and timebase <= 200ms/div.

-9  :   *WAI allowed with SINGLE only

230 :   Write error

232 :   Instrument not initialized

________________________________________________________________________________

ox8022_channel

int ox8022_channel (int channelN, int COUPling, int RANGe, int MODE);

Purpose

Panel name  :   Inputs Settings

Description :   This panel is usefull to

set the channels CH1 and

CH2 parameters.

* Coupling   :    AC,DC,GND

* Range      :    200V/DIV..10mV/DIV

* Mode       :    NOR/DIFF

Parameter List

channelN

Variable Type       int

Control label  : Channel number

Description    : This control selects the

channel of which parameters

have to be modified.

Variable type  : integer

Variable range :     1: CH1 (default value)

2: CH2

COUPling

Variable Type       int

Control label  : Coupling

Description    : This control sets the

coupling for the selected

channel.

Variable type  : integer

Variable range :  0  AC  (default value)

1  DC

2  GND

RANGe

Variable Type       int

Control label  : Range

Description    : This parameter defines the

range (Volt/div) of the

selected channel

Variable type  : integer

Variable range : 10E-3 to 200

(1,2,5 sequences)

Default value  : 1V/DIV

MODE

Variable Type       int

Control label  : Mode NOR/DIFF

Description    : Sets instrument in mode NOR

or DIFF.

Variable type  : integer

Variable range : 0 DIFF

1 NOR   (default value)

Return Value

Control label  :  Error

Description    :  Displays the result of the

function call.

Variable type  : integer

Variable range :

0  :   Success

-1  :   Channel out of range

-2  :   Coupling out of range

-3  :   Range out of range

-4  :   Mode out of range

230 :   Write error

232 :   Instrument not initialized

________________________________________________________________________________

ox8022_channel_save

int ox8022_channel_save (int SAVE, int savestate);

Purpose

Panel name  :   Channel save

Description :   This panel is usefull to

Save or not a channel.

Saving a channel prevent

from an undesirable

writing in its memory.

Parameter List

SAVE

Variable Type       int

Control label  : Save

Description    : This control selects the

channel whom save state

has to be modified.

Variable type  : integer

Variable range :    0: CH1

1: CH2

Default value  : 0  (CH1)

savestate

Variable Type       int

Control label  : Save State

Description    : This control valids or not

the save function of the

selected channel.

Variable type  : integer

Variable range :    0:  OFF (Default value)

1:  ON

Return Value

Control label  :  Error

Description    :  Displays the result of the

function call.

Variable type  : integer

Variable range :

0  :   Success

-1  :   SAVE out of range

-2  :   Save state out of range

-3  :   Acquisition Running (Select

Acquisition Pannel and Stop

it)

230 :   Write error

232 :   Instrument not initialized

________________________________________________________________________________

ox8022_close

int ox8022_close (void);

Purpose

Function name : Close

Description   : Close the instrument module

and unlock the front pannel.

Return Value

Control label  :  Error

Description    :  Displays the result of the

function call.

Variable type  : integer

Variable range :

0   :   Success

221 :   Unable to close instrument

230 :   Write error

232 :   Instrument not initialized

234 :   Error in placing instrument

in local mode.

________________________________________________________________________________

ox8022_cont_inst

int ox8022_cont_inst (int mode, int autoset);

Purpose

Panel name  :   MODE & AUTOSET

Description :   This panel  is usefull to

switch the MODE (analogical

or numeric) and to

run the AUTOSET function

(look for the right

settings to display the

signal on the screen).

Parameter List

mode

Variable Type       int

Control label  : Instrument mode

Description    : Defines the oscilloscope mode

Variable type  : integer

Variable range :

0 :  Analogical mode  (default

value)

1 :  Digital Storage mode

autoset

Variable Type       int

Control label  : AUTOSET

Description    : Run the automatic scaling

function .

Variable type  : integer

Variable range :  0  : OFF (default value)

1  : ON

NOTE :  The autoscale function analyses the input signals and

sets time base range (according to the slowest signal),vertical

range, trigger source and display mode. If no signal is present, the

instrument recalls its previous settings .

Return Value

Control label  :  Error

Description    :  Displays the result of the

function call.

Variable type  : integer

Variable range :

0  :   Success

-1  :   Mode out of range

-2  :   Autoset out of range

230 :   Write error

232 :   Instrument not initialized

________________________________________________________________________________

ox8022_display

int ox8022_display (int MODE, int HOR_EXP, int HOR_POS, int interpolation,

int filter);

Purpose

panel name  :   Display settings

description :   This panel is usefull to

configure and control the

format of the display.

The following parameters

can be set here :

Mode          :     CH1 ALT CHOP CH2 XY ADD MULT

H Exp         :     x0.2..x20

H Position    :     0..4000

Interpolation :     Off,LN,SN

Parameter List

MODE

Variable Type       int

Control label  : MODE

Description    : This control configures the

vertical display mode of the

scope.

Variable type  : integer

Variable range : 0 : CH1   (default value)

1 : ALT

2 : CHOP

3 : CH2

4 : XY

5 : ADD

6 : MULT  (*)

* MULT is not allowed in analogical scope mode.

HOR_EXP

Variable Type       int

Control label  : HOR_EXP

Description    : This control configures the

horizontal expansion in

numerical mode.

Variable type  : integer

Variable range :

x0.2, x0.5, x1, x2, x5, x10, x20

Default value  :  3  (x1)

Warning : if you have acquired a signal at a

range faster  5s/Div (2s .. 0.1S) , then :

x1  will set the time base to 2s/div

x2  will set the time base to 1s/div

x5  will set the time base to 0.5s/div

x10 will set the time base to 0.2s/div

x20 will set the time base to 0.1s/div

HOR_POS

Variable Type       int

Control label  : HOR_POS

Description    : This control configures the

window position in numerical

mode.

Variable type  : integer

Variable range : 0 to 4000

Default value  : 0

interpolation

Variable Type       int

Control label  : Interpolation

Description    : This control sets the type of

interpolation (Digital

Storage Display).

Variable type  : integer

Variable range : 0 : Off  (default value)

1 : LN

2 : SN

filter

Variable Type       int

Control label  : Filter

Description    : This control sets the Y

filtering (Digital Storage

Display)

Variable type  : Integer

Variable range : 0 : OFF    (default value)

1 : ON

Return Value

Control label  :  Error

Description    :  Displays the result of the

function call.

Variable type  : integer

Variable range :

0  :   Success

-1  :   MODE out of range

-2  :   HOR_EXP out of range

-3  :   HOR_POS  out of range

-4  :   Interpolation out of range

-5  :   Filter out of range

-6  :   MULT not available in AN mode

-7  :   HOR EXP <>3 (x1) not available

when Acquisition is running

230 :   Write error

232 :   Instrument not initialized

________________________________________________________________________________

ox8022_init

int ox8022_init (int resetDevice, int lockFrontPanel, int RS232portnumber,

int baudRate, int parity);

Purpose

Panel name : Initialize

Description :

* Open the communication RS232

with the right parameters :

serial port number

speed,parity

* Identify the device (OX8022/8032)

* Clear Status (*CLS)

* If enabled , reset the device (*RST)

* If enabled , lock the front pannel

Default Instrument Setup (after *RST) :

Memory                  :   Off

Front pannel            :   Unlocked

Communication           :   PC

Input Settings

CH1/CH2 coupling    :   AC

Mode                :   Diff

Input Range         :   0.1V/DIV

Channel Save

CH1                 :   Off

CH2                 :   Off

Acquisition Settings

Mode                :   REFRESH

Pre-trigger         :   0ko

Peak                :   Off

Run/Stop            :   Stop

Display Settings

Mode                :   CHOP

Horizontal expansion:   x1

Window position     :   0000

Interpolation       :   Off

Y filtering         :   NO

Timebase Settings

x10                 :   Off

Auto                :   On

Delay               :   Off

Range               :   50S/DIV

Trigger Settings

Mode P-P            :   Off

Slope               :   +

Source              :   CH1

Coupling            :   DC

Data format

Format              :   ASCII

Header ADIF         :   Off

Hardcopy Settings

format              :   Plot (<V2.0)

IBM Pr (>=V2.0)

Grid                :   Off

Screen sharing      :   Off

Status              :   Off

Measures Settings

Cursors             :   C1

Measure             :   Off

H cursors position  :  C1: -4 DIV

C2: +4 DIV

V cursors position  :  C1: -5 DIV

C2: +5 DIV

C3:  0 DIV

Reference           :  CH1

Parameter List

resetDevice

Variable Type       int

Control Name:  Reset

Description:   Specifies if the instrument is to

be reset to its default settings

during the initialization

procedure.

Variable Type: Integer

Valid Range:   1 = Yes (default value)

0 = No

lockFrontPanel

Variable Type       int

Control Name:  Lock Front Panel

Description:   Specifies if the Front Panel is to

be locked or unlocked during the

initialization procedure.

Variable Type: Integer

Valid Range:   1 = Yes

0 = No   (default value)

Warning : If you do not lock the front panel and

modify the setup of the instrument from

its front panel , the driver does not

know this new setup and some errors may

be wrongs or not seen .

RS232portnumber

Variable Type       int

Control label  : RS232 port number

Description    : Defines the RS232 port number

Variable type  : integer

Variable range : 1 to 8 (Default value : 1)

baudRate

Variable Type       int

Control Name:  Baud Rate

Description:   Specifies  the baud rate at which

to communicate with the

instrument.

Variable Type: Integer

Valid Values:  110,150,300,600,1200,2400,4800,

9600 bauds

Default Value: 9600

parity

Variable Type       int

Control Name:  Parity

Description:   Specifies the parity mode to

be used to communicate with the

instrument.

Variable Type: Integer

Valid Values:  0 - no parity  (default value)

1 - odd parity

2 - even parity

Return Value

Control label  :  Error

Description    :  Displays the result of the

function call.

Variable type  :  global integer

Variable range :

0  :  Success

-1  :  Reset Device out of range

-2  :  Lock Front Pannel out of range

-3  :  RS232 port out of range

-4  :  Baud Rate out of range

-5  :  Parity out of range

-6  :  Error interpreting instrument

response

(Error Code - 300) =  1 to 124

1 - 124     RS-232 error (see RS-232 Library

error codes)

220  :  Unable to open Instrument

230  :  Write Error

231  :  Read Error

233  :  Error in setting GPIB primary

address

________________________________________________________________________________

ox8022_instrument_query

int ox8022_instrument_query (int query, int n);

Purpose

Panel name  :   Instrument Queries

Description :   This panel is usefull to

read the instrument

settings.

Parameter List

query

Variable Type       int

Control label  :  Query

Description    : This control defines which

query has to be sent to the

instrument .

Variable type  : integer

Variable range : 0 to 29

Default Value  : 0 :  MODE?

n

Variable Type       int

Control label  : Channel

Description    : This control defines which

channel setup has to be

read.

(when using DISP:SAVE<N>? queries) .

Variable type  : integer

Variable range : 1:  CH1  (Default value)

2:  CH2

Return Value

Control label  :  Error

Description    :  Displays the result of the

function call.

Variable type  : integer

Variable range :

0  :   Success

-1  :   Query out of range

-2  :   Channel out of range

230 :   Write error

231 :   Read error

232 :   Instrument not initialized

________________________________________________________________________________

ox8022_measure

int ox8022_measure (int measureControl, int reference);

Purpose

Panel name  :   Automatic Measurements

Description :   This panel is useful to

run and read the automa-

tics measurements .

Measurements :

Reference:  CH1 CH2

Type     :  AC

DC

FREQuency

PERiod

PHASe

AMPLitude

LOW

HIGH

RISE:TIMe

FALL:TIMe

NWIDth

PWIDth

PDUTy

NDUTy

MINimum

MAXimum

PTPeak

Parameter List

measureControl

Variable Type       int

Control label  : Measure control

Description    : This control defines the

type of measurement .

Variable type  : integer

Variable range : The measures available are :

0       AC      (default value)

1       DC

2       FREQuency

3       PERiod

4       PHASe

5       AMPLitude

6       LOW

7       HIGH

8       RISE:TIMe

9       FALL:TIMe

10       NWIDth

11       PWIDth

12       PDUTy

13       NDUTy

14       MINimum

15       MAXimum

16       PTPeak

reference

Variable Type       int

Control label  : Reference

Description    : This control defines the

waveform to be measured.

Variable type  : integer

Variable range : 1 CH1

2 CH2

Default value :  1 (CH1)

Return Value

Control label  :  Error

Description    :  Displays the result of the

function call.

Variable type  : integer

Variable range :

0   :  Success

-1   :  Measure Control out of range

-2   :  Reference out of range

230   :  Write error

231   :  Read error

232   :  Instrument not initialized

________________________________________________________________________________

ox8022_setup

int ox8022_setup (int setup, int storage, int RAMnumber, char filename[]);

Purpose

Panel name  :   Instrument Setup

Description :   This panel is usefull to

Save or Recall the instrument

Setup to or from RAM or

file .

Parameter List

setup

Variable Type       int

Control label  :  Configuration

Description    :  Selects the Save or Recall

Instrument Setup function .

Variable type  : integer

Variable range :

0 : Save   (default value)

1 : Recall

storage

Variable Type       int

Control label  :  Configuration

Description    :  Selects the target (RAM or

File)

Variable type  : integer

Variable range :  0 : RAM   (default value)

1 : File

RAMnumber

Variable Type       int

Control label  :  RAM Number

Description    :  Selects the number of

the Setup to be stored

or recalled.

Variable type  :  integer

Variable range :  from 1 to 10

(default value : 1)

filename

Variable Type       char []

Control label  :  Filename

Description    :  Specify the filename of the

file containing the device

setup.

Variable type  :  String

Note : Syntaxe with C language : if you

specify the complete pathname , you have to

write \\ instead of \ :

c:\\temp\\setup1

Return Value

Control label  :  Error

Description    :  Displays the result of the

function call.

Variable type  :  global integer

Variable range :

0  :  Success

-1  :  Setup out of range

-2  :  Storage out of range

-3  :  RAM number out of range

-4  :  Error Opening file

-5  :  Nothing in this RAM

230  :  Write Error

231  :  Read Error

232  :  Instrument not initialized

________________________________________________________________________________

ox8022_status

int ox8022_status (int queriesCommands, int maskValue);

Purpose

Panel name  :   IEEE 488.2 Queries

Description :   This panel is usefull to

read some status of the

scope.

*ESE?   (event status enable)

*ESR?   (Event status register)

*IDN?   (Scope Identification)

*LRN?   (System setup)

*OPC?   (Operation Complete)

*SRE?   (Service request Mask)

*STB?   (Status byte)

*TST?   (Emulate a test of the scope)

Parameter List

queriesCommands

Variable Type       int

Control label  : Queries/Commands

Description    : Defines the type of IEEE

query to be sent OR sets

the Enable Register (event

or status register).

Variable type  : integer

Variable range :   0 to 9

Queries (from 0 to 7)

0 :  *ESE?   (Event status enable)

1 :  *ESR?   (Event status register)

2 :  *IDN?   (Scope Identification)

3 :  *LRN?   (System setup)

4 :  *OPC?   (Operation END)

5 :  *SRE?   (Service request Mask)

6 :  *STB?   (Status byte)

7 :  *TST?   (Emulate a test of the scope)

Commands (from 8 to 9)

8 :  *ESE<NRf> (Set Event status enable)

9 :  *SRE<NRf> (Set Event status register)

Default value : 0 (*ESE?)

maskValue

Variable Type       int

Control label  : Mask Value

Description    : Defines the Event Status

Enable and Service Request

Enable registers .

Variable type  : integer

Variable range : 0 to 255

Default Value  : 255

Return Value

Control label  :  Error

Description    :  Displays the result of the

function call.

Variable type  : integer

Variable range :

0  :   Success

-1  :   Queries/Commands out of range

-2  :   Mask Value out of range

230 :   Write error

231 :   Read error

232 :   Instrument not initialized

________________________________________________________________________________

ox8022_timebase

int ox8022_timebase (int EXPHx10, int MODE, int DELay, int RANGe);

Purpose

Panel name  :   Time base

Description :   This panel is usefull to

configure Time base

parameters :

X10         :  Off,On

Mode AUTO   :  Off,on

Delay       :  Off,Search,Delay

Range       :  200S/DIV ..0.1S/DIV      (Num)

100mS/DIV..(8022)0.5S/div (An)

(8032)0.2S/div

Parameter List

EXPHx10

Variable Type       int

Control label  : EXPH (x10)

Description    : This control configures the

horizontal x10 expanded mode

This mode is useful only

in analogical mode.

Variable type  : integer

Variable range : 0 : OFF   (default value)

1 : ON

MODE

Variable Type       int

Control label  : AUTO

Description    : This control configures the

trigger in AUTOmatic mode

or triggered mode.

Variable type  : integer

Variable range : 0 : NORMAL   (default value)

1 : AUTO

DELay

Variable Type       int

Control label  : Delay

Description    : This control configures the

delayed time base state.

Variable type  : integer

Variable range : 0 : OFF     (default value)

1 : SEARCH

2 : DELAY

RANGe

Variable Type       int

Control label  : Range

Description    : Defines the speed of the time

base.

Variable type  : integer

Variable Range :    0:    0.1S/DIV

28:     200S/DIV

(1,2,5 sequences)

Default value  :    7:    20S/DIV

NOTE :  Analogical mode  : 0.5E-6 to 200E-3 (8022)

0.2E-6 to 200E-3 (8032)

Numerical mode   : 0.1E-6 to 200

Return Value

Control label  :  Error

Description    :  Displays the result of the

function call.

Variable type  : integer

Variable range :

0  :   Success

-1  :   EXPH out of range

-2  :   MODE out of range

-3  :   DELay out of range

-4  :   RANGe out of range

-5  :   RANGe not allowed with ROLL

-6  :   RANGe not allowed with AN

-7  :   SEARCH not allowed with NUM

230 :   Write error

232 :   Instrument not initialized

________________________________________________________________________________

ox8022_trigger

int ox8022_trigger (int MODE, int SLOPe, int SOURce, int COUPling);

Purpose

Panel name  :   Trigger

Description :   This panel is usefull to

configure the trigger

parameters.

These parameters are :

P-P Mode     :   Off,On

Slope        :   +,-

Source       :   CH1 CH2 ALT LINE EXT

Coupling     :   DC  AC  LF  HF   TVH TVV

Parameter List

MODE

Variable Type       int

Control label  : Peak to peak

Description    : Defines the trigger P-P mode

Variable type  : integer

Variable range : 0 : NORM   (default value)

1 : PTP

SLOPe

Variable Type       int

Control label  : Slope

Description    : Defines the trigger slope

Positif or negatif

Variable type  : integer

Variable range : 0 : +   (default value)

1 : -

SOURce

Variable Type       int

Control label  : Source

Description    : Defines the trigger source

Variable type  : integer

Variable range : 0 : CH1    (default value)

1 : CH2

2 : ALT

3 : LINE

4 : EXT

COUPling

Variable Type       int

Control label  : Coupling

Description    : Defines the trigger input

coupling.

Variable type  : integer

Variable range : 0 : DC     (default value)

1 : AC

2 : LF

3 : HF

4 : TVH

5 : TVV

Return Value

Control label  :

Description    :  Displays the result of the

function call.

Variable type  : integer

Variable range :

0  :   Success

-1  :   MODE out of range

-2  :   SLOPe out of range

-3  :   SOURCe out of range

-4  :   COUPling out of range

230  :   Write error

232  :   Instrument not initialized

________________________________________________________________________________

ox8022_wave_read

int ox8022_wave_read (int waveform, int header, int peak, int storeinFile,

double outputArray[], char fileName[]);

Purpose

Panel name  :   Read Waveform

Description :   This panel is useful for

reading waveforms and

storing its either in

an array or in a file.

Note : About the header...

The header contains scale informations and helps

you to graduate the axis for representings the

signals . Once the Waveform Read is executed ,

the variables ox8022_xscale , ox8022_yscale and

ox8022_yoffset are updated .

ox8022_xscale represents the time interval

between two samples (in Second).

Example : if you want to display 1000 points

on the screen , the X axis length is

1000*ox8022_xscale.

ox8022_yscale represents the voltage between

two quantums (ADC 8 bits==> 256 quantums).

200 quantums are displayed on the oscilloscope

screen (+-100) (25 quantums/DIV).

Example : if you want to display 8 Divisions

on the Y axis , the minimum value for the

Y axis is (-100*ox8022_yscale) and the

maximum value is (100*ox8022_yscale).

Parameter List

waveform

Variable Type       int

Control name  : Waveform

Description   : Selects the waveform to be

read.

Variable type : integer

Valid range   :    0  CH1

1  CH2

2  S1

3  S2

where S1 represents the signal displayed on

the screen and whose parameters are displayed

at the left top corner (S2 is the other

signal (if present)).

Default value :    0   (CH1)

header

Variable Type       int

Control label  : Data Interchange Format

Description    : This control specifies if the

header is sent by the device

before the trace data .

Variable type  : integer

Variable range : 0: OFF     (default value)

1: ON

Note : the header contains the scale informations

(x,y) and the 0 Volt position. See your

Programming Reference Manual for more

informations .

peak

Variable Type       int

Control label  : Peak

Description    : This control specifies the

Peak (Min or Max) of the

channel to be read .

Variable type  : integer

Variable range : 0: Min     (default value)

1: Max

storeinFile

Variable Type       int

Control label  : Store in File?

Description    : This control specifies if the

samples are recorded in a

file .

Variable type  : integer

Variable range : 0: No      (default value)

1: Yes

outputArray

Variable Type       double []

Control label  : Output Array

Description    : Defines the array in which

samples have to be stored .

Variable type  : double array

Variable range : Any existing array name

Size :  Waveforms S1 and S2 : 1024

All others          : 4096

fileName

Variable Type       char []

Control name  : Filename

Description   : Specifies the filename (with

its pathname) to which the

samples are writen .

Variable type : String

Default value : c:\\temp\\wave.dat

Return Value

Control name : Count

Description  : Returns the number of bytes

read .

Variable type: integer

Valid range  : Waveform S1,S2: 1024

All others    : 4096