OX822 Metrix

________________________________________________________________________________

Introduction:

This instrument driver provides programming support for OX822 Metrix.

It contains functions for opening, configuring, taking measurements from, and

closing the instrument.

________________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

For RS-232 instrument drivers:

-  the instrument is connected to the RS-232 interface.

-  the COM port, baud rate, parity, and timeout supplied to the

initialize function must match the settings of the instrument.

________________________________________________________________________________

Error and Status Information:

Each function in this instrument driver returns a status code that

either indicates success or describes an error or warning condition.

Your program should examine the status code from each call to an

instrument driver function to determine if an error occurred.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

The description of each instrument driver function lists possible error

codes and their meanings

________________________________________________________________________________

How To Use This Document:

Use this document as a programming reference manual.

It describes each function in the

OX822 Metrix

instrument.  The functions appear in alphabetical order, with a description

of the function and its C syntax, a description of each parameter, and a list

of possible error codes.

________________________________________________________________________________

Function Tree Layout:

OX822 Metrix                           Function Name:

Initialize                                  ox822_init

Instrument Setup                            ox822_setup

Configure

Display settings                         ox822_display

Channel settings                         ox822_channel

Time base                                ox822_timebase

Trigger                                  ox822_trigger

Close                                       ox822_close

________________________________________________________________________________

OX822 Metrix

Instrument name: OX822 Metrix

Description: This instrument module provides

RS232 support for OX822.

The module contains the followings

functions:

(1) Initialize

Initialize the instrument module

with default values and lock the

front panel.

(2) Instrument setup

(3) Instrument Configuration

Class of functions for configuring

the following parameters:

* Display settings

* Channel settings

* Time Base

* Trigger

(4) Close

Close the instrument module and

unlock the front panel.

________________________________________________________________________________

The following functions are in alphabetical order.

________________________________________________________________________________

ox822_channel

int ox822_channel (int channel, int coupling, int range, int input);

Purpose

Panel name  :   Channel settings

Description :   Set the Channel , its

coupling and volts/division .

Set the Input NOR DIFF .

Coupling :  AC DC GND

Volts/Div:  10  mV/Div  to 200 V/Div

with 1-2-5 sequences .

Input    :  NOR DIFF

Parameter List

channel

Variable Type       int

Control label  : Channel number

Description    : Specifies the channel of the

oscilloscope to configure.

Variable type  : integer

Variable range :    0: CH1 (default value)

1: CH2

coupling

Variable Type       int

Control label  : Coupling

Description    : Set the coupling for the

selected channel.

Variable type  : integer

Variable range :    0 :  AC (default value)

1 :  DC

2 :  GND

range

Variable Type       int

Control label  : Range

Description    : Set the range (V/Div)

of the specified channel

Variable type  : integer

Variable range :    13   :  10   mV/Div

12   :  20   mV/Div

11   :  50   mV/Div

10   :   0.1  V/Div

9   :   0.2  V/Div

8   :   0.5  V/Div

7   :   1    V/Div (default value)

6   :   2    V/Div

5   :   5    V/Div

4   :  10    V/Div

3   :  20    V/Div

2   :  50    V/Div

1   : 100    V/Div

0   : 200    V/Div

input

Variable Type       int

Control label  : Input NOR DIFF

Description    : Sets the instrument in mode NOR

or DIFF.

Variable type  : integer

Variable range :    0 : DIFF

1 : NOR  (default value)

Return Value

Control label  :  Error

Description    :  Displays the result of the

function call.

Variable type  :  global integer

Variable range :

0  :  Success

-1  :  Channel value out of range

-2  :  Coupling value out of range

-3  :  Range value out of range

-4  :  Input NOR DIFF value out

of range

230   :  Error writing to instrument

232   :  Instrument not initialized

Others :  See rs232err values

________________________________________________________________________________

ox822_close

int ox822_close (void);

Purpose

Pannel name : Close

Description   : Close the instrument module

and unlock the front panel.

NOTE: The instrument must be re-

initialized to use it again.

Return Value

Control label  :  Error

Description    :  Displays the result of the

function call.

Variable type  : integer

Variable range :

0 :       Success

221 :       Unable to close instrument

232 :       Instrument not initialized

________________________________________________________________________________

ox822_display

int ox822_display (int verticalMode);

Purpose

Panel name  :   Display settings

Description :   This panel is usefull to

configure the Vertical mode.

Vertical Mode : CH1 ALT CHOP ADD CH2

Parameter List

verticalMode

Variable Type       int

Control label  : Vertical Mode

Description    : Sets the vertical mode

Variable type  : integer

Variable range : 0 : CH1 (default value)

1 : ALT

2 : CHOP

3 : ADD

4 : CH2

Return Value

Control label  :  Error

Description    :  Displays the result of the

function call.

Variable type  : integer

Variable range :

0  :       Success

-1  :       Vertical Mode out of

range

-2  :       Beam Finder out of range

230   :  Error writing to instrument

232   :  Instrument not initialized

Others :  See rs232err values

________________________________________________________________________________

ox822_init

int ox822_init (int RS232portnumber);

Purpose

Panel name : Initialize

Description : Opens the instrument

with the following RS232

configuration :

9600 Bauds,8 bits,no parity

1 stop bit, XON/XOFF

and set the instrument to

default values.

Parameter List

RS232portnumber

Variable Type       int

Control label  : RS232 port number

Description    : Defines the RS232 port number

Variable type  : integer

Variable range : 1 to 8

Default value : 1

Return Value

Control label  :  Error

Description    :  Displays the result of the

function call.

Variable type  :  global integer

Variable range :

0   :  Success

-1   :  Invalid port number RS232

-2   :  Invalid Lock front panel number.

223   :  Instrument identification

query failed.

230   :  Error writing to instrument

Others   :  See rs232err

________________________________________________________________________________

ox822_setup

int ox822_setup (int setup, char filename[]);

Purpose

Panel name  :   Instrument Setup

Description :   This panel is usefull to

Save or Recall the instrument

Setup to or from a file .

This file is in ASCII format and

contains the values of the

functions listed below :

Parameter List

setup

Variable Type       int

Control label  :  Configuration

Description    :  Selects the Save or Recall

Instrument Setup function .

Variable type  : integer

Variable range :    0 : Save (default value)

1 : Recall

filename

Variable Type       char []

Control label  : File name

Description    : Defines the File name which

contains the Instrument

Setup.

Variable type  : string

Variable range : any valid string

(use \\\ with Quick C in case

of complete pathname :

Example  : "c:\\\lw\\\programs\\\example")

Return Value

Control label  :  Error

Description    :  Displays the result of the

function call.

Variable type  :  global integer

Variable range :

0  :  Success

-1  :  Setup value out of range

-2  :  File doesn't exist

230   :  Error writing to instrument

231   :  Error reading from instrument

232   :  Instrument not initialized

Others :  See rs232err values

________________________________________________________________________________

ox822_timebase

int ox822_timebase (int x10, int range, int delay);

Purpose

Panel name  :   Time base

Description :   This panel is usefull to set

the Time base parameters of

the oscilloscope :

x10       :  Off On

Range     :  200 ms/Div to 0.5S/Div

with 1-2-5 sequences

Delayed TB:  Off Search Delayed

Parameter List

x10

Variable Type       int

Control label  : x10

Description    : This control configures the

horizontal x10 expanded mode

On or Off .

Variable type  : integer

Variable range :    0 : Off (default value)

1 : On

range

Variable Type       int

Control label  : Range

Description    : Set the timebase Range of

the oscilloscope .

Variable type  : integer

Variable range :    0   :   200 mS

1   :   100 mS

2   :   50  mS

3   :   20  mS

4   :   10  mS

5   :   5   mS

6   :   2   mS

7   :   1   mS

8   :   0.5 mS

9   :   0.2 mS

10  :   0.1 mS  (default value)

11  :   50  S

12  :   20  S

13  :   10  S

14  :   5   S

15  :   2   S

16  :   1   S

17  :   0.5 S

18  :   XY

delay

Variable Type       int

Control label  : Delay

Description    : This control configures the

delayed time base state.

Variable type  : integer

Variable range :    0 : Off (default value)

1 : Search

2 : Delay

Return Value

Control label  :  Error

Description    :  Displays the result of the

function call.

Variable type  : integer

Variable range :

0   :      Success

-1   :      x10 value out of range

-2   :      Range value out of range

-3   :      Delay value out of range

-4   :      Vertical Mode = XY

230  :  Error writing to instrument

232  :  Instrument not initialized

________________________________________________________________________________

ox822_trigger

int ox822_trigger (int ptp, int slope, int auto, int source, int coupling);

Purpose

Panel name  :   Trigger

Description :   This panel is usefull to

configure the following

trigger parameters:

- Peak to Peak : on/off

- Slope : + / -

- Auto  : on/off

- Source : CH1,CH2,LINE,

ALT,EXT

- Coupling : AC,DC,LF,HF,

TVH,TVV

Parameter List

ptp

Variable Type       int

Control label  : Ptp

Description    : Set the trigger Peak to Peak

mode Off or On .

Variable type  : integer

Variable range :    0 : Off

1 : On (default value)

slope

Variable Type       int

Control label  : Slope

Description    : Set the trigger slope

Positive or negative

Variable type  : integer

Variable range :    0 : Positive (default value)

1 : Negative

auto

Variable Type       int

Control label  : Auto

Description    : Sets the trigger Auto mode

Off or On .

Variable type  : integer

Variable range :    0 : Off

1 : On (default value)

source

Variable Type       int

Control label  : Source

Description    : Sets the trigger Source .

Variable type  : integer

Variable range : 0 : CH1  (default value)

1 : CH2

2 : ALT

3 : LINE

4 : EXT

When Trigger Source LINE is configured, Trigger

Coupling is disabled.

coupling

Variable Type       int

Control label  : Coupling

Description    : Sets the trigger Coupling .

Variable type  : integer

Variable range : 0 : DC (default value)

1 : AC

2 : LF

3 : HF

4 : TVH

5 : TVV

When Trigger Source LINE is configured, Trigger

Coupling is disabled.

Return Value

Control label  :  Error

Description    :  Displays the result of the

function call.

Variable type  : integer

Variable range :

0  :     Success

-1  :     Ptp value out of range

-2  :     Slope value out of range

-3  :     Auto value out of range

-4  :     Source value out of range

-5  :     Coupling value out of range

-6  :     Vertical Mode = XY

230  :     Error writing to instrument

232  :     Instrument not initialized